# CHRONO-1 — Preregistration

Series: CHRONO (temporal nonclassicality) Experiment ID: CHRONO-1 Owner: Remnant Fieldworks Inc. (Derek Adam Hone) Status at lock: design + harness complete, validated in simulation, hardware job pending Governing framework: RF Experiment–Pilot–IP Covenant (preregistration-first; honest outcomes; claims narrower than evidence; file-first / publish-after for new matter)

## 1. One-sentence statement

We measure the three-time Leggett–Garg inequality (temporal Bell-type inequality) on a single superconducting qubit and cryptographically bind the measured K3 statistic into an authorization ProofRecord as a temporal nonclassicality witness — a certificate that the authorized value had no definite pre-measurement (pre-execution) history under the stated assumption.

## 2. Why this is new (distinct from all prior RF series)

| Series | Phenomenon tested | Resource witnessed |
| --- | --- | --- |
| ARK | authorization boundary (context replay) | exact-match policy |
| WITNESS | provenance of a superposition-sourced nonce | assumed unpredictability |
| BELLWETHER | spatial nonclassicality (CHSH Bell, S>2) | Bell violation bound into nonce |
| CHRONO | temporal nonclassicality (Leggett–Garg K3>1) | time-ordered nonclassicality bound into nonce |

No prior work (RF or, to our knowledge, anyone) binds a Leggett–Garg temporal-correlation witness into an authorization/ProofRecord construction. BELLWETHER witnessed nonclassicality across space; CHRONO witnesses it across time — the physical statement most directly aligned with RF doctrine: Verification Before Execution (the value did not exist definitely before it was verified).

## 3. Physics and design

Single qubit, dichotomic observable Q = Z (outcome +1 for |0⟩, −1 for |1⟩). Between measurement times the qubit precesses under R_y(θ) with θ = π/3 (60°).

Three two-time correlators, each measured in a separate circuit using mid-circuit measurement:

C12 : measure(t1) -> R_y(θ) -> measure(t2)
C23 : R_y(θ) -> measure(t2) -> R_y(θ) -> measure(t3)
C13 : measure(t1) -> R_y(2θ) -> measure(t3)

Each circuit writes two bits into one 2-bit classical register c (c[0] = earlier, c[1] = later). Correlator = shot-average of the product of the two signed outcomes.

Leggett–Garg K3: K3 = C12 + C23 − C13

- Macrorealist bound (H1): K3 ≤ 1

- Quantum (Lüders) bound: K3 = 1.5 (maximal at θ = π/3)

- Ideal predictions: C12 = C23 = cos60° = +0.5, C13 = cos120° = −0.5, so K3 = 1.5.

Simulation validation (pre-lock, noiseless AerSimulator, 200k shots/correlator): C12 = 0.502, C23 = 0.503, C13 = −0.504, K3 = 1.508 — consistent with the ideal 1.5; macrorealist bound violated at ~152σ. The counts-extraction path is confirmed identical to the hardware path.

## 4. Execution plan

- Backend: IBM open-plan hardware (prefer ibm_fez, else min pending-jobs).

- Shots: 2000 per correlator × 3 = 6000 shots total (~a few seconds of QPU time; sized to the remaining open-plan budget).

- Mode: Batch (open plan forbids Session). Channel: ibm_quantum_platform.

- Circuits transpiled to backend ISA (optimization_level=1) before submission.

## 5. Outcome criteria (fixed in advance — honest, symmetric)

| Verdict | Condition |
| --- | --- |
| VIOLATION (temporal nonclassicality, under assumptions) | K3 > 1 and K3 − 1 > 3σ |
| INCONCLUSIVE / HOLD | K3 > 1 but not 3σ-significant |
| NO VIOLATION | K3 ≤ 1 |

The measured hardware K3 will be reported exactly as observed, whichever bin it lands in. No re-runs to chase a preferred number.

## 6. Honesty boundary (non-negotiable — carried into every artifact)

- The Leggett–Garg inequality assumes non-invasive measurability (NIM): that a macrorealist system can be read without disturbance. A projective qubit measurement is invasive, so a determined macrorealist can attribute any K3 > 1 to measurement disturbance — the “clumsiness loophole.”

- This is therefore a Leggett–Garg violation UNDER STATED ASSUMPTIONS, not a loophole-free refutation of macrorealism, and not device-independent certified randomness.

- Single system, single device: no device-independence is claimed.

- Only defensible claim: “observed temporal correlations violating the macrorealist bound K3 ≤ 1 on hardware, under the non-invasive-measurability assumption, cryptographically bound into the authorization record.”

### 6a. Disclosed pre-execution harness facts

- Local simulator (StatevectorSampler) cannot execute mid-circuit measurements; validation used AerSimulator. The hardware path (SamplerV2, Batch) executes mid-circuit measurement natively. This does not alter any circuit or claim.

- Secrets key renamed witness1_ibmq → ibm quantum; the token loader tries both (FIX-3). No physics impact.

## 7. ProofRecord construction (chrono-proofrecord-1.0)

raw_counts_hash  = H( canonical(counts_by_correlator) )
calibration_hash = H( canonical(calibration_snapshot) )
witness_hash     = H( canonical(temporal_witness) )     # contains K3, correlators, bounds
chrono_nonce     = H( lp(raw_counts_hash) || lp(job_id) || lp(calibration_hash) || lp(witness_hash) )
record_hash      = H( canonical(all fields except record_hash) )

Tampering with the measured statistic breaks chrono_nonce; any covered-field edit breaks record_hash. Verified by 9/9 pre-lock mock tests (tamper-evidence over both counts and witness) and by the independent verify.py.

## 8. Commercial purpose (Covenant requirement)

CHRONO extends the ExecutionProof patent family with a temporal entropy/authorization primitive: a ProofRecord whose seed is certified to have possessed no definite value prior to its verification event. Candidate RF-100 use: anti-replay / freshness certificates in time-ordered authorization (a token that provably could not have been precomputed). Publication is gated pending the file-first IP determination (§9).

## 9. IP gate

CHRONO-1 is almost certainly new patentable matter. Per the Covenant it is held local-only (no GitHub / Zenodo / LinkedIn / RF-100 disclosure) until Derek makes the file-first determination. Execution is not disclosure; running the job and fetching results is permitted.

To God be the glory. Proof Before Power. Verification Before Execution.